MIR SUITE

System Technical Documentation

Dual-arm mobile manipulation platform
MiR200 · 2× UR5e · BrainCo Revo1 · Orbbec Gemini · Nordbo
Tampere University (TUNI) / Fastlab

Version 1.0 — July 2026

# Table of Contents

1. Background and Initial System State

2. Containerization Process

3. MiR Communication Bridge Integration

4. User Interface

5. Operating Guide

6. Technical Reference Manual

Appendices

# 1. Background and Initial System State

The MIR Suite project originated from work developed by Eemil, a laboratory researcher who implemented the original control system for the robotic platform. The system was distributed across three main repositories located in the otros_proyectos_eemil/ directory:

| Project | Description | Main Content |
| --- | --- | --- |
| pbd_system/ | Programming by Demonstration system | ROS workspace (~5 GB) with duo_ur packages, moveit_utils_pkg, contact_graspnet, SAM3 segmentation module and Tilix automation scripts |
| pandai_ark/ | Perception and navigation system | Initialization scripts, Polyscope extension modules and network utilities |
| Universal_Robots_ROS2_Driver/ | Official UR driver | Vendored source code of the official ROS 2 driver for Universal Robots arms |

The original implementation required direct execution of all processes on the Jetson AGX Orin operating system, without isolation or containerization mechanisms. The ROS workspace was located at the absolute path ~/pbd_system/ros_ws/, with hardcoded dependencies and paths that prevented environment reproducibility on other machines.

## 1.1 Identified Limitations

Communication between the MiR200 mobile base and the ROS 2 environment was performed using the mir_raw.py script, which established a connection with the MiR internal rosbridge and republished all available topics (approximately 150). This behavior caused saturation of the DDS domain shared with the UR5e arm controllers.

Unnecessary occupation of the DDS bus with diagnostic topics and internal MiR200 states

Loss of critical messages from the UR5e controllers

Error 9000 on the MiR200 (safety PLC activation and emergency stop) when transitioning to Running mode

General system instability, particularly in the arm controllers

All processes —Python scripts, ROS nodes and Bash routines— ran directly on the host operating system without any container or virtual machine. The system lacked a unified graphical interface, requiring SSH connections and manual script execution for any robot operation.

## 1.2 Restructuring Objectives

Based on the analysis of identified limitations, the following objectives were defined:

Containerization of all components using Docker to ensure isolation and reproducibility

Reduction of DDS traffic through a selective bridge with the MiR200 base

Development of a unified web graphical interface with authentication and access control mechanisms

Implementation of a security policy based on the principle of least privilege

Complete documentation of the architecture and operational procedures

# 2. Containerization Process

## 2.1 Component-Based Reorganization

As the first stage of the restructuring process, all source code was reorganized into a hierarchical component-based structure. Each robot subsystem was assigned to a specific directory within mir_suite/:

| Directory | Component | Responsibility |
| --- | --- | --- |
| UR/ | UR5e arms | ROS 2 driver, joint server, motion manager, action bridge |
| MiR/ | MiR200 mobile base | Selective communication bridge, scanner fusion, supervision mechanisms |
| PERIPHERAL/ | Peripherals | Orbbec Gemini 335Lg camera driver |
| ENDEFFECTOR/ | End effectors | BrainCo Revo1 hand control, touch sensor |
| UI/ | User interface | FastAPI backend, Svelte 5 frontend, Caddy TLS proxy, mDNS publication |
| BATTERY_MS/ | Battery/BMS | Research and documentation (phase 2) |
| HEAD_NECK/ | Motorized neck | Research and documentation (phase 2) |
| ROUTER_MODEM/ | Teltonika RUTX50 router | 5G connectivity configuration |

## 2.2 Docker Service Architecture

The entire system was defined in a single docker-compose.yml file declaring 7 services. All containers use network_mode: host to share a common ROS 2 communication domain:

| Service | Container | Profile | Function |
| --- | --- | --- | --- |
| caddy | mir_caddy | always | TLS/HTTP reverse proxy; sole LAN listening point |
| mdns | mir_mdns | always | Publication of mir-suite.local via avahi |
| mir_ui | mir_ui | always | FastAPI backend and compiled web UI; docker.sock access |
| camera | mir_camera | always | Orbbec driver (started on demand as a node) |
| mir | mir_mir | always | Selective MiR bridge, scanner fusion and supervision |
| ur_driver | mir_ur_driver | arms, full | UR driver, rosbridge, joint server, action bridge |
| ur_driver_sim | mir_ur_driver_sim | sim | Same image with simulated hardware for testing |

Key architectural characteristics:

Single ROS graph: all instances share ROS_DOMAIN_ID=75, resulting in approximately 41 topics visible from any container

Shared /dev/shm: FastDDS requires shared memory between processes on the same host

Docker profiles: arms (real hardware) and sim (simulation) selectable via --profile

No container operates with elevated privileges: device access granted through specific cgroup rules

Note: /dev/shm must be explicitly shared. Without this configuration, topics appear listed but do not transmit data between containers. Symptom: "topic exists, zero messages".

## 2.3 Security Policy and Least Privilege

A policy of removing unnecessary privileges was applied to all containers. Instead of the privileged: true directive, only the minimum required permissions were granted:

| Container | Previous privileges | Current privileges | Mechanism |
| --- | --- | --- | --- |
| mir_ur_driver | privileged: true | SYS_NICE + IPC_LOCK | cap_add + rtprio/memlock ulimits |
| mir_camera | privileged: true | c 189:* rmw | device_cgroup_rules + /dev/bus/usb |
| mir_mir | privileged: true | none additional | network_mode: host sufficient |
| mir_ui | privileged: true | docker.sock | exclusive docker socket mount |

This modification was validated on real laboratory hardware. The UR5e arm controller operates correctly with SYS_NICE and IPC_LOCK for its real-time loop.

## 2.4 Network Infrastructure and Encryption

A two-tier Public Key Infrastructure (PKI) was implemented to ensure confidentiality of all web interface communications:

Local root Certificate Authority with 10-year validity

CA private key stored exclusively on the Jetson (mode 600, excluded from version control)

Server certificate signed by the local CA with 825-day validity

Subject Alternative Names: mir-suite.local, tunisite.local, lab.local, localhost, 192.168.1.75, 127.0.0.1

mDNS publication of mir-suite.local via avahi

Caddy proxy terminating TLS and routing traffic to uvicorn and rosbridge

# 3. MiR Communication Bridge Integration

## 3.1 DDS Domain Saturation Problem

The mir_raw.py script established a connection with the MiR200 internal rosbridge (192.168.1.14:9090) and republished all topics published by the mobile base into the local ROS 2 domain. The MiR200 exposes approximately 150 topics, including internal diagnostics, component states, odometry and raw sensor data.

This unnecessary traffic saturated the DDS domain shared with the UR5e arm controllers, causing Error 9000 on the MiR200 (safety PLC and emergency stop) and significant degradation in communication with the UR5e controllers.

## 3.2 Adopted Solution: Selective Topic Bridge

During analysis of this problem, a selective bridge implementation was identified in the fortis-uc1-tau (TK32) repository developed by Wael. This implementation solved the problem by transmitting only the essential topics for system operation. On July 10, 2026, this solution was integrated into the codebase, completely replacing mir_raw.py.

The implemented bridge (mir_bridge.py) uses the roslibpy library to connect to the MiR200 rosbridge and transmits exclusively the following topics:

| Direction | Topics | Frequency | Purpose |
| --- | --- | --- | --- |
| MiR → ROS 2 (subscribe) | /f_scan | ~13 Hz | Front SICK laser scanner |
| MiR → ROS 2 (subscribe) | /b_scan | ~13 Hz | Rear SICK laser scanner |
| MiR → ROS 2 (subscribe) | /odom | 41 Hz | MiR200 base odometry |
| MiR → ROS 2 (subscribe) | /tf | - | Transforms (with map→odom filter) |
| MiR → ROS 2 (subscribe) | /tf_static | - | Static transforms |
| MiR → ROS 2 (subscribe) | /MC/battery_percentage | - | Battery status |
| ROS 2 → MiR (publish) | /cmd_vel | - | Base velocity (PLC-supervised) |

The bridge deliberately filters the map→odom transform published by the MiR200, allowing a future local SLAM system to assume localization responsibility.

## 3.3 Supervision and Recovery Mechanisms

The mir container entrypoint manages the bridge lifecycle through multiple supervision layers:

| Component | Function |
| --- | --- |
| Liveness check (mir_liveness.py) | Subscribes to /odom and updates a heartbeat file with each received message |
| Watchdog (mir_watchdog.sh) | Every 15s checks heartbeat; after 25s without activity, terminates the bridge process |
| Reconnection (mir_entrypoint.sh) | Exponential backoff loop (10s → 20s → 40s → 60s maximum) |
| Scanner fusion (scanners_merger.py) | Executed outside the reconnection loop, independent of MiR200 status |

Implementation results: drastic reduction of DDS traffic (from approximately 150 topics to 7), verified system stability and absence of Error 9000 since production deployment.

# 4. User Interface

The unified graphical interface was developed to replace terminal-based operation. It consists of a FastAPI backend and a Svelte 5 frontend, communicating through an authenticated REST API and WebSockets for real-time telemetry.

## 4.1 Backend Architecture

The backend resides in a single file (UI/backend/main.py, 1604 lines) implementing all server logic via FastAPI, uvicorn, docker-py, httpx and WebSockets.

Login screen.

### API Endpoints

| Area | Route | Method | Purpose |
| --- | --- | --- | --- |
| Authentication | /api/login, /api/logout, /api/me | POST/POST/GET | Session management via tokens |
| Profile | /api/me/config, /api/me/apply | PUT/POST | Configuration and start saved nodes |
| Users | /api/users | GET/POST/PUT/DELETE | User management (administrators) |
| Containers | /api/containers | GET/POST | Docker Compose service control |
| Nodes | /api/nodes | GET/POST/GET | ROS node management and log query |
| System | /api/system | GET | Infrastructure process status |
| UR | /api/ur/status¦start¦stop¦joints¦move¦payload¦freedrive | GET/POST | UR driver control and telemetry |
| MiR | /api/mir/status | GET | Mobile base status via REST API |
| Force | /api/force | GET/POST | Nordbo force/torque sensors |
| Terminal | /api/term | WebSocket | Container Bash terminal (administrators only) |

Backend security measures:

Authentication tokens with 24-hour TTL stored in volatile memory

Passwords protected via PBKDF2-SHA256 with 200,000 iterations

Rate limiting: 8 consecutive failures result in a 300-second lockout

Timing attack protection via dummy hash for non-existent users

pkill pattern validation through restrictive regular expression

can_control flag per user enabling or restricting control actions

## 4.2 Frontend Architecture

The frontend was developed with Svelte 5 and Vite, compiled and embedded in the backend Docker image. It consists of 12 Svelte components and 13 reactive stores:

| Component | Route | Functionality |
| --- | --- | --- |
| Overview.svelte | #/overview | Dashboard with status indicators, joints and force gauges |
| UrPanel.svelte | #/arms | Joint telemetry, motion controls and skills |
| SkillsPanel.svelte | #/arms | Skills execution (grasp, move) |
| ForcePanel.svelte | #/arms | Nordbo force sensor readings |
| MirPanel.svelte | #/mir | MiR200 base status |
| CameraPanel.svelte | #/camera | Live video stream via rosbridge |
| SystemView.svelte | #/system | Node, container and process manager |
| TerminalPanel.svelte | #/terminal | Container Bash terminal |

## 4.3 Communication Flow

Data exchange between the browser and system follows this path:

Browser —— https/http ——> Caddy (:443/:80)

├── /rosbridge  ——> rosbridge (localhost:9090, read-only)

└── other routes ——> uvicorn (localhost:8080)

│  FastAPI (main.py)

├── docker exec ——> container scripts

├── http ——> joint_server (localhost:9091)

├── http ——> MiR REST (192.168.1.14:80)

└── ws ——> Nordbo sensors (:2003)

Real-time telemetry reaches the browser via subscription through rosbridge. Commands never traverse rosbridge but are sent through the authenticated REST API.

## 4.4 Views Description

Overview: system status indicators.

The main view presents the overall system status: active containers, real-time bar graphs of both arm joints (400 Hz), Nordbo force sensor readings and the experimental data recording button.

Arms: joint telemetry and motion controls.

The Arms panel allows monitoring each joint position, executing incremental movements, activating manual guidance mode, setting payload and running predefined skills.

MiR: mobile base status.

Camera: live video stream.

System: node and container manager.

Rosbag node information.

Web terminal in container.

Camera USB management panel.

# 5. Operating Guide

## 5.1 System Access

The system is accessible from any device connected to the laboratory network:

| URL | Description |
| --- | --- |
| http://192.168.1.75 | Unencrypted access from any device on the laboratory network |
| https://mir-suite.local | Encrypted access; requires installation of the laboratory CA |

## 5.2 Startup Procedures

cd /home/lab/Desktop/MIR/mir_suite

docker compose up -d                            # Base services

docker compose --profile arms up -d             # + real arms

docker compose --profile sim up -d              # Simulation

docker compose down                             # Stop all

In the user interface, access System to start the required nodes. For the arms, power on the robots at their control panels; the system operates in headless mode without requiring the Play button.

## 5.3 UR5e Arm Control

Both UR5e arms (left: 192.168.1.102, right: 192.168.1.103) are controlled by a single ur_ros2_control_node instance. Key parameters: headless_mode=True, reverse_ip=192.168.1.75, tf_prefix left_/right_, state publication at 400 Hz.

## 5.4 Data Flow Verification

ros2 topic list | wc -l         # ~41 topics

ros2 topic hz /odom             # ~41 Hz with MiR200 powered on

ros2 topic hz /joint_states     # ~400 Hz with UR driver active

ros2 topic hz /camera/color/image_raw/compressed   # ~30 Hz

## 5.5 Software Update

git pull

docker restart mir_ui              # Backend only

docker build --network=host -f UI/ui_image/Dockerfile -t mir_ui:latest .

docker compose up -d --force-recreate mir_ui

# 6. Technical Reference Manual

This chapter reproduces content from the MIR_SUITE_MANUAL.md technical manual, verified against the production system on July 13, 2026.

## 6.1 System Specifications

| Component | Model | Address |
| --- | --- | --- |
| Computer | NVIDIA Jetson AGX Orin (64 GB) | 192.168.1.75 |
| Mobile base | MiR200 | 192.168.1.14 |
| Left arm | Universal Robots UR5e | 192.168.1.102 |
| Right arm | Universal Robots UR5e | 192.168.1.103 |
| Hands | 2× BrainCo Revo1 | /dev/hand_L, /dev/hand_R |
| Camera | Orbbec Gemini 335Lg | /dev/bus/usb |
| Force/torque | 2× Nordbo NRS-6200 | .112 (left), .113 (right) |
| Router | Teltonika RUTX50 (5G) | 192.168.1.1 |

## 6.2 Jetson Ports

| Port | Access | Service |
| --- | --- | --- |
| 80, 443 | LAN | Web UI + API + /rosbridge + /ca.crt |
| 8080 | localhost | uvicorn (behind Caddy) |
| 9090 | localhost | rosbridge (behind Caddy) |
| 9091 | localhost | joint_server (arm telemetry) |
| 50001–50004 | LAN | Left arm reverse interface |
| 50011–50014 | LAN | Right arm reverse interface |

## 6.3 Security

| Measure | Detail |
| --- | --- |
| No privileged containers | Device access via cgroup rules |
| LAN surface = Caddy only | uvicorn, rosbridge and joint_server on localhost |
| Rosbridge read-only | Topic whitelist; services and actions denied |
| Authentication + can_control | Tokens with 24h TTL; per-user control flag |
| Command hardening | pkill pattern whitelist, per-node locks |
| TLS with private CA | Two-tier PKI; CA key never leaves the Jetson |
| Secrets out of git | .env, *.key, *.pem in .gitignore |
| Default passwords changed | admin/wael/pablo no longer use admin/wael/pablo; now Fastlab-2026/! (pbkdf2-sha256 hash, 200k iterations) |
| Terminal: mandatory second factor | Strong, unique SHELL_PASSWORD in config/.env; terminal is disabled on a default; constant-time comparison (hmac.compare_digest) |

# 7. Recent Updates (2026-07-24)

## 7.1 Access Security

The default passwords of the three UI accounts (admin, wael, pablo) were changed from their factory values to Fastlab-2026/!. Profiles are stored hashed (pbkdf2-sha256, 200000 iterations) in UI/backend/data/profiles/*.yaml; config/.env was updated for any future re-seed.

The terminal second factor (SHELL_PASSWORD) is kept as a strong random value, distinct and independent from the login, stored only in config/.env.

## 7.2 Web Terminal Hardening

• The /api/term endpoint now requires a strong SHELL_PASSWORD: if the value is empty or a known default (fastlab2026, changeme, password, admin) the terminal is disabled and a warning is logged at boot.

• The password comparison uses hmac.compare_digest (constant time), preventing timing side-channel leaks.

• The Connect button now opens a modal dialog to enter the password (paste is reliable) instead of the old header field.

• Black-box verification (2026-07-24): no token, invalid token, non-admin user (wael/pablo), wrong password and a non-whitelisted container are all rejected before a shell opens. The only valid path is admin token + correct SHELL_PASSWORD + allowed container.

## 7.3 Emergency Stop and Arm Control

• Software E-STOP: /api/estop endpoint and a red button in the top bar; it actually cancels active action goals (stop_arm) on both arms and both hands, and disables freedrive.

• Wrist validation in MoveRequest (allow_wrist, default False) and a terminal exec_resize cap (max 200x200) to avoid Docker daemon hangs.

## 7.4 System Robustness

• Healthchecks on all six services (caddy, mdns, mir_ui, camera, mir, ur_driver/ur_driver_sim) and memory/CPU limits on every container.

• UR driver startup: verify that control ports 30002 and 30004 are open on 192.168.1.102 and 192.168.1.103 (ping alone is not enough) before starting the driver. If launched with the arms powered off, the node aborts (RTDE) and leaves a stale lock; the clean recovery is docker restart mir_ur_driver followed by a single Start from the UI.

# Appendices

## A. Repository Layout

mir_suite/

├── docker-compose.yml       # 7 services

├── config/                  # .env, tls/

├── UI/                      # backend/, web/, caddy/, mdns/

├── UR/                      # UR5e arms

├── MiR/                     # MiR200 mobile base

├── PERIPHERAL/              # Orbbec camera

├── ENDEFFECTOR/             # BrainCo hands

├── BATTERY_MS/             # Battery (phase 2)

├── HEAD_NECK/              # Neck (phase 2)

├── ROUTER_MODEM/           # Teltonika router

├── documentation/          # This document

├── logs/                   # Logs and rosbags

└── robot_workspace/        # Vendored ROS workspace

## B. Environment Variables

| Variable | Value | Description |
| --- | --- | --- |
| ROS_DOMAIN_ID | 75 | Shared ROS 2 domain |
| MIR_IP | 192.168.1.14 | MiR200 IP address |
| LEFT_ARM_IP | 192.168.1.102 | Left UR5e arm |
| RIGHT_ARM_IP | 192.168.1.103 | Right UR5e arm |
| TOKEN_TTL | 86400 | Token TTL (24h) |
| NORDBO_LEFT_IP | 192.168.1.112 | Left Nordbo sensor |
| NORDBO_RIGHT_IP | 192.168.1.113 | Right Nordbo sensor |
| NORDBO_PORT | 2003 | Nordbo WebSocket port |
| UR_MAX_DELTA | 0.5 | Max jog delta (rad) |
| SHELL_PASSWORD | (secret) | Web terminal second factor; required and unique, distinct from login |

## C. Reference Commands

# Deployment

docker compose up -d

docker compose --profile arms up -d

docker compose down

# Logs

docker compose logs -f mir_ui

docker compose logs -f mir_mir

docker compose logs -f mir_ur_driver

# Container shell

docker exec -it mir_ur_driver bash

docker exec -it mir_mir bash

docker exec -it mir_camera bash

# ROS debug

ros2 topic list

ros2 topic hz /odom

ros2 topic hz /joint_states

ros2 control list_controllers

## D. Architecture Diagram

## E. System Reference Table

| Item | Value | Description |
| --- | --- | --- |
| Jetson IP | 192.168.1.75 | Main computer |
| mDNS | mir-suite.local | Local network name |
| MiR200 | 192.168.1.14 | Mobile base |
| Left UR5e | 192.168.1.102 | Left arm |
| Right UR5e | 192.168.1.103 | Right arm |
| Left Nordbo | 192.168.1.112 | Left F/T sensor |
| Right Nordbo | 192.168.1.113 | Right F/T sensor |
| Router | 192.168.1.1 | Teltonika RUTX50 |
| WiFi SSID | Robot_suite | 2.4 GHz, channel 11 |
| REST API | http://192.168.1.75/api | FastAPI backend |
| Web UI | https://mir-suite.local | Graphical interface |
| Admin | admin, wael, pablo / Fastlab-2026/! | UI passwords (changed from default on 2026-07-24) |
| ROS_DOMAIN_ID | 75 | ROS 2 domain |
| Repository | github.com/fjmp-dev/TUNI-PROJECT | Branch: refactor/by-component |